RTL Issues – AXI FIFO BFM

## Issue 1: AWCACHE and ARCACHE Signal Mismatch with AXI Spec

Description: AWCACHE[3:0] and ARCACHE[3:0] are not aligned with AXI specification, causing issues while sending transactions to slave.

Impact: Transactions fail or behave unexpectedly.

Affected Files: TOP file, AXI_master.sv, AXI_MASTER_WRITE_CONTROL.sv, AXI_MASTER_READ_CONTROL.sv

Recommendation: Update cache signal encoding as per AXI4 spec across all mentioned files.

## Issue 2: Decoder Does Not Support Multiple Write/Read Packets

Description: When two write packets are sent from write FIFO, decoder treats second packet as continuation of first.

Impact: Multiple packets are merged incorrectly into one transaction.

Affected Files: decoder.sv

Recommendation: Fix SOP/EOP detection logic to correctly identify packet boundaries.

## Issue 3: Decoder Treats EOP as WDATA and Incorrect Data Slicing

Description: EOP is incorrectly considered as wdata and 16 bits, 64-bit, 128 bits data slicing is not done properly.

Impact: Corrupted data sent to AXI master.

Affected Files: decoder.sv

Recommendation: Exclude EOP from data payload and correct slicing logic.

## Issue 4: Missing Read Response Packet Construction Logic

Description: Decoder has only read response input ports but no logic to build read response packet.

Impact: Read data is lost .

Affected Files: decoder.sv

Recommendation: Implement read response packet assembly and push to read FIFO.

## Issue 5: AXI Master Repeats Read Transaction for Same Address

Description: rd_trn_en never deasserts causing FSM to re-enter Addr_State.

Impact: Duplicate read transactions for same address.

Affected Files: AXI_MASTER_READ_CONTROL.v

Recommendation: Deassert rd_trn_en after transaction completion and gate state transitions.

Waveform of Axi read control:

## Issue 6: ARVALID Asserted Before Valid Address and Control Signals

Description: ARVALID asserted early and RREADY asserted same cycle as RVALID, violating AXI timing.

Impact: Slave interprets address as 0x00000000 and returns incorrect data.

Affected Files: AXI_MASTER_READ_CONTROL.v

Recommendation: Align ARVALID with valid address/control and assert RREADY one cycle before RVALID.